PATERN MATCH UPDATE

STEP 1.

Runt test process to allow test process to capture UUT logo image.

C:\SNAPSHOT.BMP

STEP 2.

Open Vision Assistant 2014.

STEP 3.

Open file C:\SNAPSHOT.BMP

Open Script File C:\mtp\Site_Ne_sch\Projects\DS99_FAT\LED_LOGO_DETECT\TIMON LED AND LOGO VALIDATION_PATTERN_MATCH .vascr

Set Image on screen to “Zoom to Fit”

Step 4.

Edit Pattern Matching LOGO step

Create New Template image:

Save Image file C:\mtp\Site_Ne_sch\Test_Templates\IMAGES\ NCR_Logo.png

STEP 5.

Confirm Vision Assistant detect new Logo

STEP 6.

Close Vision Assistant 2014.

Add NCR model 2356-2308-0000 to test datablock file” C:\mtp\Site_Ne_sch\Projects\DS22XX_FAT\DS22XX_FAT.DataBlocks to support the logo type based on the model number.